## Capteur SRF08

Ce module, idéalement conçu pour les applications de télémétrie pour robotique  est capable de déterminer la distance qui le sépare d'un obstacle se présentant devant lui (entre 3 cm et 6 m).

Il s'interface à l'aide de son bus I2C™ .

Il comporte aussi un un capteur de lumiere aussi.

Caractéristiques techniques:
- Alimentation: + 5 Vcc.
- Consommation: 15 mA env. - 3 mA (au repos).
- Angle de détection: 55 ° env.

- Fréquence: 40 kHz

- Portée de mesure: de 3 cm à 6 m (réglable)

-Unité de mesure: mm, pouces ou µs

- Gain variable .

brochage :

Datasheet :https://www.cs.york.ac.uk/micromouse/Docs/SRF08UltraSonicRanger.pdf

#include<Wire.h>

void setup(){

Wire.begin();

//I2c bus

Serial.begin(115200);

delay(100);

}

void loop(){

int distance=getDistance();

Serial.print("Distance:");

Serial.print(distance);

Serial.print("cm");

delay(100);

}

int getDistance()                            //fonction retourant la distance

{

int reading=0;

// step 1: instruct sensor to read echoes

Wire.beginTransmission(112); // transmit to device #112 (0x70)

// the address specified in the datasheet is 224 (0xE0)

// but i2c adressing uses the high 7 bits so it's 112

Wire.write(byte(0x00));      // sets register pointer to the command register (0x00)

Wire.write(byte(0x51));      // command sensor to measure in "centimeters" (0x51)

// use 0x50 for inches

// use 0x52 for ping microseconds

Wire.endTransmission();      // stop transmitting

// step 2: wait for readings to happen

delay(70);                   // datasheet suggests at least 65 milliseconds

// step 3: instruct sensor to return a particular echo reading

Wire.beginTransmission(112); // transmit to device #112

Wire.write(byte(0x02));      // sets register pointer to the first echo  register (0x02)

Wire.endTransmission();      // stop transmitting

// step 4: request reading from sensor

Wire.requestFrom(112, 2);    // request 2 bytes from slave device #112

// step 5: receive reading from sensor

if (2 <= Wire.available()) { // if two bytes were received

reading = Wire.read();  // receive high byte (overwrites previous reading)

reading = reading << 8;    // shift high byte to be high 8 bits

reading |= Wire.read(); // receive low byte as lower 8 bits

return(reading);

//or

//byte highByte = Wire.read();                 // Get high byte

//byte lowByte = Wire.read();                  // Get low byte

// reading= (highByte << 8) + lowByte;

}

}